# Experiment #2 – Diodes and Applications

# Youssef Samwel

# yo800238@ucf.edu

# EEE3307 Electronics I

Section 0014

# Due Date 10/1/2023

# Project Description

# The students were tasked with constructing and analyzing diode-based circuits. The purpose of experiment is to expose the students to application of diodes and types of usage in circuits.

# 2.0 About Laboratory Day

# The laboratory session took place on the Monday section between 6:00pm and 8:50pm on September 25, 2023. My lab partners were Nicolas and Brandon.

# 3.0 Computer Simulation (SPICE)

# Pre-Laboratory Simulations

A) For the circuits of Fig. 1 and Fig. 2, choose available values of RL and C so that RLC = 0.2 second approximately. Draw the output waveforms when the input is sinusoidal of frequency 100 Hz and 10 V peak to peak, under the following cases:

- Capacitor only is removed. Plot the transfer (Vout versus Vin) characteristics.

- Resistor only is removed.

- Capacitor and resistor are both in place. Calculate the peak-to-peak ripple voltage.

Figure 1

Figure 2

Waveform without Capacitor

Waveform without Resistor

Waveform with both Capacitor and Resistor

To compute the ripple voltage, we use the half-way rectifier equations, as shown below:

R=2kΩ;C=100µF;f=100 Hz;Vmagnitude=5 V

Vripple=VmfRC→5100*2k*100µF

Vripple=250 mV

The simulated ripple voltage in LT-spice is 189 mV.

The difference between the simulated value and the computed value is that the equation assumes that the diode has ideal behavior, however, in the simulation, LT-spice accounts for diode non-ideal parameters.

B) Determine the transfer characteristics of the circuits in Fig. 3 and Fig. 4. Draw the output waveforms assuming that the input is a sinusoid with sufficiently larger amplitude (larger than both reference voltages or the Zener voltages) amplitude.

Figure 3

Figure 4

Input vs Output Voltage for Fig. 3

For Vb we used 1.5 V and the resistance value was 2kΩ.

Input vs Output Voltage for Fig. 4

The breakdown voltage for the Zener diodes in the simulation is 6.2 V which is similar to the diode used in the experiment.

C) For the circuit shown in Fig. 5, draw the output waveform if the input is a sinusoid. Do not neglect the diode turn on voltage (≈ 0.65 V). Select available values of R and C so that RC time constant is equal to 0.2 seconds approximately (e.g., for a capacitor of 10 µF, the resistor value should be 20 kΩ).

Figure 5

Input vs Output Voltage for Fig. 5

R=2kΩ;C=100µF;f=100 Hz;Vpp=10 V

# 4.0 Experiment Procedure

# 5.0 Observations and Simulation Comparison

# 6.0 Learned Objectives

- Use of function generator

- Measurement using oscilloscope and practical probing.

- Simulation via LT-spice

- Diode performances and diode IV curve

- Half-wave rectifier

- DC current measurement

# 7.0 Conclusion