# Virtual Lab 3 – Waveguide

# Youssef Samwel

# yo800238@ucf.edu

# EEL4436C Microwave Engineering

Section 0012

# Due Date 10/28/2023

Rectangular Waveguide

- (10 pts) Plot the E field magnitude on the top surface of the waveguide for 6, 10, and 15 GHz. From the field plot, determine whether it is a propagating mode or evanescent mode at each frequency.

Magnitude of Electricfield at 6 GHz

Magnitude of Electricfield at 10 GHz

Magnitude of Electricfield at 15 GHz

From the plot it can be seen that at 6 GHz the wave is in evanescent mode but for both 10 and 15 GHz modes the wave is in propagating mode.

- (10 pts) Calculate the guided wavelength λg at 10 GHz using the equation in the book and compare with the guided wavelength read from the field plots.

The following MATLAB code was used to compute the guided wavelength,

e_0 = 8.854187817e-12; % C²/(N * m^1)

u = 4*pi*1e-7;

w = (10*10^9)*2*pi;

k = w * sqrt(u*e_0);

a = 22.86e-3;

b = 10e-3;

m = 1;

n = 0;

kc = sqrt( (m*pi/a)^2 - (n*pi/b)^2 );

beta = sqrt(k^2 - kc^2);

guided_wavelength = 2*pi/beta; % meters

The computed guided wavelength is 39.7 mm.

The measured guided wavelength in HFSS is 40mm. The simulated value is exactly the same as the computed value, the reason for minor differences is due to the low resolution of the measuring position tool in HFSS.

Measured Guided Wavelength

- (10 pts) Calculate how many modes can exist at 18 GHz and identify the name of each mode.

Five modes can exist at 18 GHz and the following is the name of each mode.

| MODE | n | m | Cut off (GHz) |
| --- | --- | --- | --- |
| TE | 0 | 1 | 6.56 |
| TE | 0 | 2 | 13.12 |
| TE | 1 | 0 | 15.00 |
| TE/TM | 1 | 1 | 16.37 |

- (10 pts) Plot the vector E and H fields of each mode and provide the name of the mode in the plot.

EL TE01

EL TE10

EL TE11

EL TE20

EL TM11

ME TE01

ME TE10

ME TE11

ME TE20

ME TM11

- (10 pts) Plot the vector electric current on the top surface and one side wall for TE10 mode at 10 GHz. What is the relationship between the current and the magnetic field for this mode? If you want to plot the fields for the higher-order mode, go to “HFSS” “Fields” “Edit Sources” and change the amplitude of the higher-order mode to be 1 and set the amplitude of other modes to be 0.

- (10 pts) There is no unique definition of impedance for a waveguide which does not support TEM mode. To figure out which impedance is used in HFSS simulation, first read the port 1 impedance at 8, 10 and 12 GHz and then identify the formula for the impedance calculations in HFSS.

The closest impedance equation to match port impedance is the ZTM=βηκ.

Coaxial Line:

- (10 pts) What is the cutoff frequency of the first higher-order mode? Which mode is it?

The first higher mode for a coaxial line is TE11 and to compute the higher order mode cut-off frequency of a coaxial line we use the following equations from the Pozar book,

kc=2a+b

fc=ckc2πer

The cut off frequency for this waveguide is 11.78 GHz, the following is the MATLAB code used to compute the cutoff frequency.

%% coaxial cut off frequency

c = 299792458;

a = 1.27e-3; % meter (original is 50mil)

b = 4.318e-3; % meter (original is 170mil)

k_c = 2 / (a + b);

e_r = 2.1; % teflon

% cut off frequency for TE11

f_c = (c*k_c)/(2*pi*sqrt(e_r));

f_c = f_c/1e9;

- (10 pts) Plot the vector E and H field distribution of the TEM and the first higher order mode.

TEM Mode

TE11 Mode

Circular Waveguide

- (10 pts) What is the cutoff frequency of the lowest and 1st higher-order modes? Which modes are they?

The cutoff frequency of the lowest mode (TE11) is 17.57 GHz, and the cutoff frequency of the next higher order mode is 22.95 GHz (TM01).

| MODE | n | m | Cut off (GHz) |
| --- | --- | --- | --- |
| TE | 0 | 1 | 36.58 |
| TE | 1 | 1 | 17.57 |
| TE | 2 | 1 | 29.14 |
| TE | 0 | 2 | 66.95 |
| TE | 1 | 2 | 50.87 |
| TE | 2 | 2 | 64.00 |
| TE | 0 | 3 | 97.08 |
| TE | 1 | 3 | 81.46 |
| TE | 2 | 3 | 95.14 |
| TM | 0 | 1 | 22.95 |
| TM | 1 | 1 | 36.57 |
| TM | 2 | 1 | 49.00 |
| TM | 0 | 2 | 52.68 |
| TM | 1 | 2 | 66.95 |
| TM | 2 | 2 | 80.32 |
| TM | 0 | 3 | 82.58 |
| TM | 1 | 3 | 97.09 |
| TM | 2 | 3 | 110.89 |

- (10 pts) Plot the vector E and H field distribution of the two modes.

Electric field TE11

Magnetic field TE11

Electric field TM01

Magnetic field TM01