HW4

- Consider a dipole antenna with a/λ = 0.006 at 600 MHz.

  - Find its resonant length and resonant resistance using the plots. Plot R and X over 470-750 MHz using Tai’s formula.

  - This antenna is now placed in a corner reflector of 90o angles at a distance s =0.5λ from the apex of the corner at 600 MHz.

    - Calculate the new Zin of the antenna.

    - Change the antenna length so as to cancel the reactance of the images. What is the new resonant length and resistance. (Normally, you should be two iterations, but let us just do one)

    - Calculate the Zin of this antenna at ±10% bandwidth (540, 660 MHz) and the resulting VSWR.

- Answer the following questions in short clear sentences. Not more than half a page for each questions.

  - Which antenna will have more gain (all antennas have the same total length (L > 4λ): A traveling-wave antenna? A Yagi-Uda antenna? A log-periodic dipole array?

  - Assume you have an antenna sending a circularly-polarized wave and a short dipole receiver with a power detector (diode). How would you know if the polarization is left handed or right handed?

  - Design a log periodic dipole antenna for 50 MHz to 800 MHz operation. Label all relevant dimensions. Estimate the directivity.

  - What limits the bandwidth of a planar spiral or log-periodic antenna (low frequency and high frequency limits)

  - What happens to the directivity, beamwidth and sidelobes of an array pattern if reverse tapering is done on the array? Assume the array is >>5λ.

- Consider a 13-element array on the z-axis with elements at -6d, -5d, …, 0, … , +5d, +6d. The spacing between the elements, d, is 0.62λ.

  - Use a raised cosine square taper distribution and get a pattern with a sidelobe of -27±1 dB. Plot the pattern with no phase distribution and determine its HPBW, FNBW and directivity.

  - Now calculate the phase distribution over the array so that it scans to θ=55o. Plot the pattern and determine its HPBW, FNBW and directivity. Are the sidelobe levels the same as the case in (a)?

  - Assume that you have a 3-bit phase shifter with l linear gain difference of -2dB between the 0 degree bit and 315 degree bit (the 315 degree bit has 2 dB more loss than the 0 degree bit and this loss is linear with phase bit). For a scan angle θ=55o, calculate:

    - The far-field pattern error due to the digitized phase shifter.

    - The far-field pattern error due to the amplitude variation.

    - Add (a) and (b) to the ideal pattern obtained in (b) and get the final pattern in dB. determine its HPBW, FNBW, SLL and directivity. If you have a choice of getting a 4-bit phase shifter or a 3-bit phase shifter with less amplitude variation, which one will you choose to satisfy your SLLs?