Polar Coding:

| BW | 2Rb(12-wid 4x BWmin) or Rb(1-wid, 2x BWmin) |
| --- | --- |
| Power | Most efficient |
| DC | Not zero (but can be made zero w/ Manchester) |

ON-OFF coding:

| BW | Same as Polar |
| --- | --- |
| Power | 2x Power of Polar (or 4x Energy) |
| DC | Not zero |

Bipolar (single bit error)

| BW | Rb (No half-width pulse) |
| --- | --- |
| Power | 2x Power (3dB) |
| DC | DC Null |

Nyquist 1st Criteria ISI

sinc gives Rb/2 BW but falls off at 1/t, other functions have faster roll-off but (1+r)Rb/2 BW. Raised cos when r = 1,falls off at 1/t^3.

Controlled ISI

ZFE

|  | Multiamplitude | Orthogonal |
| --- | --- | --- |
| Rate | log2M | log2M |
| BW | Independent of M | M |
| SNR (power) | M2 | Independent of M |

PSK requires 3dB less power than ASK(on/off). BW(FSK) > BW(ASK or PSK). Can’t use coherent demodulation for PSK but can for DPSK. DPSK 1 is same as previous bit and 0 is opposite.

FSK m-array, power const but BW increases with M linearly

PSK